Table 4.3-8:  Proposed Project Construction Air Emissions

| Year 2016 | Maximum Daily Construction Emissions, lbs/day |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Segment A | ROG | CO | NOx | SOx | PM10 | PM2.5 |
| Construction Equipment | 1.50 | 9.71 | 19.15 | 0.02 | 0.56 | 0.50 |
| Construction Truck Trips | 0.20 | 0.88 | 2.96 | 0.01 | 3.77 | 0.86 |
| Worker Trips | 1.91 | 29.73 | 2.55 | 0.07 | 1.62 | 0.68 |
| Helicopter | 10.91 | 43.51 | 15.36 | 1.11 | 11.68 | 11.68 |
| Fugitive Dust (Unmitigated) | - | - | - | - | 6.56 | 1.01 |
| Subtotal | 14.52 | 83.83 | 40.02 | 1.21 | 24.18 | 14.72 |
| Segment B | ROG | CO | NOx | SOx | PM10 | PM2.5 |
| Construction Equipment | 7.70 | 47.16 | 62.89 | 0.08 | 3.56 | 3.17 |
| Construction Truck Trips | 0.62 | 2.32 | 10.47 | 0.02 | 16.21 | 3.58 |
| Worker Trips | 0.59 | 9.20 | 2.55 | 0.02 | 0.50 | 0.21 |
| Fugitive Dust (Unmitigated) | - | - | - | - | 16.02 | 3.37 |
| Subtotal | 8.91 | 58.68 | 75.91 | 0.12 | 36.29 | 10.32 |
| Segment C | ROG | CO | NOx | SOx | PM10 | PM2.5 |
| Construction Equipment | 2.90 | 14.55 | 20.20 | 0.03 | 0.94 | 0.84 |
| Construction Truck Trips | 0.03 | 0.14 | 0.45 | 0.00 | 0.59 | 0.14 |
| Worker Trips | 0.61 | 9.50 | 2.55 | 0.02 | 0.52 | 0.22 |
| Fugitive Dust (Unmitigated) | - | - | - | - | 0.41 | 0.06 |
| Subtotal | 3.55 | 24.19 | 23.20 | 0.05 | 2.45 | 1.25 |
| Segment D | ROG | CO | NOx | SOx | PM10 | PM2.5 |
| Construction Equipment | 0.33 | 2.11 | 3.69 | 0.00 | 0.14 | 0.12 |
| Construction Truck Trips | 0.03 | 0.12 | 0.31 | 0.00 | 0.31 | 0.08 |
| Worker Trips | 0.61 | 9.50 | 2.55 | 0.02 | 0.52 | 0.22 |
| Fugitive Dust (Unmitigated) | - | - | - | - | 22.71 | 4.77 |
| Subtotal | 0.97 | 11.73 | 6.55 | 0.03 | 23.68 | 5.19 |
| Maximum Daily Emissions, 2016 | 27.95 | 178.44 | 145.69 | 1.41 | 86.61 | 31.48 |

| Table 4.3-8 (cont.): Proposed Project Construction Air Emissions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Year 2017 | Maximum Daily Construction Emissions, lbs/day |  |  |  |  |  |
| Segment A | ROG | CO | NOx | SOx | PM10 | PM2.5 |
| Construction Equipment | 9.68 | 53.81 | 80.06 | 0.10 | 3.04 | 2.71 |
| Construction Truck Trips | 0.32 | 1.43 | 4.80 | 0.01 | 0.42 | 6.01 |
| Worker Trips | 0.30 | 4.60 | 0.39 | 0.01 | 0.25 | 0.10 |
| Helicopter | 1.03 | 4.50 | 4.50 | 0.31 | 3.25 | 3.25 |
| Subtotal | 11.32 | 64.34 | 89.76 | 0.44 | 6.96 | 12.07 |
| Segment B | ROG | CO | NOx | SOx | PM10 | PM2.5 |
| Construction Equipment | 1.88 | 18.74 | 30.33 | 0.04 | 0.89 | 0.79 |
| Construction Truck Trips | 0.25 | 1.18 | 3.82 | 0.01 | 0.31 | 4.60 |
| Worker Trips | 0.34 | 5.21 | 0.45 | 0.01 | 0.28 | 0.12 |
| Subtotal | 2.47 | 25.14 | 34.59 | 0.06 | 1.48 | 5.51 |
| Segment D | ROG | CO | NOx | SOx | PM10 | PM2.5 |
| Construction Equipment | 3.91 | 20.36 | 34.15 | 0.04 | 1.14 | 1.02 |
| Construction Truck Trips | 0.12 | 0.54 | 1.81 | 0.00 | 0.17 | 2.29 |
| Worker Trips | 0.18 | 2.76 | 0.24 | 0.01 | 0.15 | 0.06 |
| Subtotal | 4.22 | 23.66 | 36.19 | 0.05 | 1.47 | 3.37 |
| Maximum Daily Emissions, 2017 | 18.01 | 113.14 | 160.54 | 0.55 | 9.91 | 20.95 |
| Notes: ROG = reactive organic gases; NOX = nitrogen oxides; CO = carbon monoxide; SOX = sulfur oxides; PM10 = particulate matter, up to 10 microns; PM2.5 = particulate matter, up to 2.5 microns. Refer to Appendix 4.3-A, Air Quality Construction Emissions, for assumptions used in this analysis, including quantified emissions reduction by control measures. Controlled emissions calculated assuming standard fugitive dust control measures, including watering the site three times daily, as SDG&E’s ordinary construction restrictions require. |  |  |  |  |  |  |

Table 4.3-9:  Criteria Air Pollutant Emissions from Operation and Maintenance

| Emissions Source | Pollutant (pounds per day) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ROG | NOx | CO | SOx | PM10 | PM2.5 |
| Daily Emissions | 0.26 | 3.13 | 1.30 | 0.01 | 1.32 | 0.34 |
| SDAPCD Thresholds | 75 | 250 | 550 | 250 | 100 | 55 |
| Is Threshold Exceeded? | No | No | No | No | No | No |

Table 4.3-10:  Greenhouse Gas Construction Emissions

|  | GHG Emissions (metric tons[MT]) |  |  |  |
| --- | --- | --- | --- | --- |
|  | CO2 | CH4 | N2O | CO2e |
| Total GHG Emissions | 1,488 | 0.12 | 0.73 | 1,717 |
| Global Warming Potential | 1 | 21 | 310 | - |
| CO2 Equivalent | 1,488 | 3 | 226 | 1,717 |
| CO2 Equivalent Total | 1,717 |  |  |  |

Table 4.3-11:  Greenhouse Gas Operational Emissions

|  | Annual GHG Emissions (metric tons[MT]) |  |  |  |
| --- | --- | --- | --- | --- |
|  | CO2 | CH4 | N2O | CO2e |
| Total Operational GHG Emissions | 4 | 0.00 | 0.00 | 4 |
| Global Warming Potential | 1 | 21 | 310 | - |
| CO2 Equivalent | 4 | 0 | 0 | 4 |
| CO2 Equivalent Total | 4 |  |  |  |