October 9, 2025

Mr. Andrew Chan

Project Manager – Energy Division

California Public Utilities Commission

505 Van Ness Avenue

San Francisco, CA 94102-3298

Re:	Response to Data Request #5 DR5-2 for the SCE TLRR Eldorado-Pisgah-Lugo 220 kV Project (A.23-04-009) – Health Risk Assessment

Dear Mr. Chan:

Southern California Edison Company (SCE) on April 21, 2023, submitted a Certificate of Public Convenience and Necessity (CPCN) application and Proponent’s Environmental Assessment (PEA) to the California Public Utilities Commission (CPUC) Energy Division for the TLRR Eldorado-Pisgah-Lugo (EPL) 220 kV Project (Project).The CPUC requested an updated HRA from SCE in Data Request #5, dated September 18, 2025. This memorandum responds to Request DR5-2 by providing an updated screening HRA for staging yards near sensitive receptors (Updated HRA) and a discussion of screening HRA methodology and results. The Updated HRA includes an analysis of new staging yard locations that have been proposed for the Project since the previous HRA memorandum, submitted on October 6, 2023. The new staging yard locations reviewed for health risk impacts include the two staging yards on parcel 057-203-109 (Cima Road Tree Square and Cima Road Rock Circle Staging Yards), one staging yard on parcel 054-449-117 (Kelbaker Road 4 Material Staging Yard), and one staging yard on parcel 054-433-105 (Kelbaker Road 1 Material Staging Yard). The staging yards on parcels 054-449-117 and 054-433-105 are located greater than 1,000 meters from any sensitive receptors and health risk impacts from these staging yards are therefore expected to be less than significant. Data Request #5 also mentions two staging yards on parcel 054-433-104, which were initially included in the list of staging yards but have been removed after being deemed unnecessary and are no longer proposed for the Project.

# HRA Background

The Office of Environmental Health Hazard Assessment’s (OEHHA) Air Toxics Hot Spots Program Risk Assessment Guidelines state that only construction activities lasting longer than two months in duration should be assessed for health risk impacts. (OEHHA, 2015) The Updated HRA therefore includes all staging yards near sensitive receptors where the duration of activities that could generate air toxics are anticipated to occur for more than two months. The OEHHA defines sensitive receptors as residences, including private homes, condominiums, apartments and living quarters; schools, including preschools and daycare centers; health facilities such as hospitals, retirement and nursing homes, long term care hospitals, hospices; in addition to dormitories or similar live-in housing where children, chronically ill individuals or other sensitive persons could be exposed to toxic air contaminants (TACs). The list of staging yards that meet these criteria is provided in Table 1.

Table 1: Staging Yards Near Sensitive Receptors

| Staging Yard Location | Count of Staging Yards | Distance to Nearest Sensitive Receptor |
| --- | --- | --- |
| Bear Valley (SR-18 and Joshua Rd.) | 1 | 201 meters (0.125 miles) |
| Lugo 1 | 1 | 120 meters (0.075 miles) |
| Lugo 2 | 1 | 500 meters (0.311 miles) |
| Cima Road Tree Square | 1 | 71 meters (0.043 miles) |
| Cima Road Rock Circle | 1 | 151 meters (0.094 miles) |

In order to evaluate health risks to sensitive receptors near activities at these staging yards, screening HRAs were completed in accordance with OEHHA guidelines. The screening risk assessment was used to determine Maximum Individual Cancer Risk (MICR) and non-cancer Acute and Chronic Hazard Index (HIA and HIC, respectively) for each staging yard listed in Table 1. The AERSCREEN dispersion model was used to determine pollutant concentrations at receptor locations. AERSCREEN is a screening level air dispersion model that generates air dispersion factors using meteorological conditions, such as ambient temperatures, wind speeds, and local terrain and surface characteristics. AERSCREEN models the estimated maximum concentration of an emitted pollutant at varying receptor distances from the emission source.

The screening HRA methodology utilizes the following information to determine risk levels:

Maximum annual emissions of TAC identified as carcinogens or non-cancerous chronic health hazards;

Maximum concentration of an emitted TAC modeled by AERSCREEN;

Maximum hourly emissions of TACs with acute health risks;

Distance from emission sources to the nearest off-site sensitive or worker receptor; and

Project characteristics such as geographic location, meteorological conditions, operating schedule, and emission release heights.

# Emissions of Toxic Air Contaminants

The operation of diesel-fueled construction equipment and vehicles at staging yards is expected to generate diesel particulate matter (DPM) emissions. Although trace quantities of other TACs could be emitted from activities such as vehicle fueling, these emissions are considered negligible and therefore have no effect on HRA results. DPM emissions were approximated using the California Emissions Estimator Model® (CalEEMod) results provided in Appendix B of the PEA. The CalEEMod model was set up with the Project portioned into 11 phases, where Phase 02: Staging Areas accounts for activities at staging yards for the entirety of the project. The Staging Areas portion of Table 3.6-1 of the PEA provides a detailed accounting of the relevant equipment and operating schedules.

The construction emissions modeled using CalEEMod for staging yards includes exhaust particulate matter emissions with aerodynamic diameters less than 10-microns and 2.5-microns (exhaust PM10 and exhaust PM2.5, respectively) from combustion equipment. As a conservative estimate, DPM was assumed to be represented entirely by exhaust PM10, as PM2.5 is a subset of PM10. The CalEEMod output for Phase 02: Staging Areas indicates 0.082 pounds (lbs) of exhaust PM10 per day from activities at staging yards. By assuming a weekly schedule of six days per week, the total annual DPM emissions from all staging yards is calculated as 25.584 lbs DPM/year.

SCE identified five (5) locations where a staging yard is located near sensitive receptors and expected to be active for at least two months. To determine DPM emissions from each individual staging yard SCE assumed that staging yard activity is independent of staging yard size (square footage) and location along the Project path. Thus, total emissions were distributed evenly between staging yards by dividing the number of staging yards at a location by the total number of staging yards for the Project and applying that percentage to the total DPM emissions.  For example, there is one (1) staging yard on SR-18 and Joshua Rd. (Bear Valley) near sensitive receptors, which equates to 1 out of 13, or 7.69%. DPM emissions allocated to each staging yard are presented in Table 2.

Table 2: Staging Yard DPM Emissions

| Staging Yard Location | Count of Staging Yards near Sensitive Receptors | Total Count of Staging Yards for Project | Percent of Project (%) | Annual DPM lbs/year |
| --- | --- | --- | --- | --- |
| Bear Valley (SR-18 and Joshua Rd.) | 1 | 13 | 7.69% | 1.968 |
| Lugo 1 | 1 |  | 7.69% | 1.968 |
| Lugo 2 | 1 |  | 7.69% | 1.968 |
| Cima Road Tree Square | 1 |  | 7.69% | 1.968 |
| Cima Road Rock Circle | 1 |  | 7.69% | 1.968 |

# AERSCREEN Modeling Parameters and HRA Methodology

The AERSCREEN air dispersion model was utilized for the screening HRA.  AERSCREEN is the screening model recommended by OEHHA and the United States Environmental Protection Agency (USEPA) and is based on the AERMOD air dispersion model.  The AERSCREEN model produces estimates of "worst-case" 1-hour concentrations for single source emissions without the need for hourly meteorological data.  It includes conversion factors to estimate "worst-case" 3-hour, 8-hour, 24-hour, and annual concentrations. AERSCREEN is intended to produce concentration estimates that are equal to or greater than the estimates produced by AERMOD with a fully developed set of meteorological and terrain data, but the degree of conservatism will vary depending on the application.

AERSCREEN, like most air dispersion models, uses mathematical formulations to characterize the atmospheric processes that disperse pollutants emitted by a source.  Using emission rates, exhaust parameters, terrain characteristics, and other parameters, AERSCREEN calculates down-wind pollutant concentrations at specified receptor distances.  The results are then used to determine compliance with National Ambient Air Quality Standards (NAAQS) and other regulatory requirements.  The results can also be used as inputs into health risk assessment calculations.

AERSCREEN can be used to model a variety of emission sources, such as a single point source, capped stack, horizontal stack, rectangular area, circular area, flare, or volume source. The Project’s staging yards were modeled as rectangular area sources. All five staging yards have irregularly shaped boundaries, but since AERSCREEN cannot model irregularly shaped area sources, rectangles that most closely approximate each staging yard boundary and area were used. AERSCREEN automatically defaults to omitting terrain and stack effects on air dispersion for rectangular area sources. The model assumes that the local terrain and associated receptors are placed on a flat plane. The AERSCREEN rectangular area source model also does not consider emission stack release parameters such as stack temperature and release velocity. Since terrain effects and stack release parameters generally decrease the modeled maximum concentration of pollutants at a receptor by increasing calculated air dispersion, this setup provides a conservative model of DPM emissions dispersion from each staging yard. Additionally, the terrain in the vicinity of each modeled staging yard is generally flat with few significant elevation changes.  As such, terrain data was not used, and the flat terrain default for area sources was utilized.

To define a rectangular area source, AERSCREEN accepts the coordinates associated with the bottom left corner of the area to be modeled, followed by inputs for length in the x-direction and length in the y-direction. The area can also be rotated relative to north by entering an offset angle. AERSCREEN defaults to an offset angle of zero degrees, which corresponds to no rotation relative to north. The combination of these parameters defines the boundary of each rectangular area source. Along with the area source boundary, the setup also requires entries for average emission height and emission rate. An average emission height of 5 feet from the ground was assumed for all five staging yards. An emission rate of 1.0 gram/second of DPM was used, which would result in modeled down-wind pollutant concentrations that are unitized, and can be multiplied by the actual pollutant emission rate (in grams/second) to obtain the final pollutant concentration values. AERSCREEN evenly distributes the input emission rate across the total area of the source, at the specified release height. As a result, the modeled down-wind pollutant concentration values are dependent on the area of the area source.

As mentioned above, rectangles were used to approximate each staging yard boundary . For Lugo 1 and Lugo 2, the staging yard boundaries are relatively well defined by rectangles. However, the Bear Valley staging yard boundary shape resembles a parallelogram and is not easily defined by a rectangle of similar shape and area. For the Bear Valley staging yard, the rectangular area source was defined such that the area of the rectangle closely matched the actual area of the staging yard boundary. However, the resulting rectangle’s boundary would be further from the nearest receptor than the actual staging yard boundary if the bottom left corner coordinates of the staging yard were used. To compensate, the bottom left corner coordinate of the rectangular area was shifted closer to the nearest receptor, along the staging yard boundary. A similar process was used for the Cima Road Tree Square and Cima Road Rock Circle staging yards to define the rectangular boundary in AERSCREEN. Figures showing the staging yard boundaries and approximations of the rectangular area sources defined in AERSCREEN are provided in Appendix A. The inputs for defining each rectangular area source are provided in Appendix B.

The land uses in the vicinity of the staging yards were evaluated to determine if the urban or rural dispersion option should be used for the air dispersion modeling.  Since urban areas typically have considerably more surface roughness as well as structures and surfaces that absorb heat, atmospheric dispersion can be somewhat different compared to rural areas.  The Project’s rural dispersion option was determined to be appropriate, as the locations are in sparsely populated/developed areas.

Representative meteorological parameters were selected in AERSCREEN for each staging yard.  AERSCREEN does not use actual meteorological data files from nearby weather stations; instead, it uses a fully developed set of default meteorological conditions.  Default parameters that can be adjusted include minimum and maximum temperatures, climate type, wind speeds, and surface friction. Based on these parameter inputs, AERSCREEN automatically generates default values for other relevant meteorological parameters. The modeling runs included the use of standard regulatory default options, except as noted.

The HRA methodology described in OEHHA’s Air Toxics Hot Spots Program Risk Assessment Guidelines was used for determining MICR and HIC for DPM emissions from the staging yards shown in Table 2. Acute (non-cancer, HIA) health risks were not assessed because there are no published acute inhalation Reference Exposure Levels (REL) for DPM. The equations used for calculating sensitive receptor cancer risk are equations 5.4.1.1 and 8.2.4 A in the OEHHA guidelines. The equations used for calculating worker receptor cancer risk are equations 5.4.1.2- A and 8.2.4 B in the OEHHA guidelines. Detailed calculations are provided in Attachment B.

# Distance to Receptors

The shorter of the distance between a staging yard boundary or AERSCREEN rectangular area source boundary and the nearest receptor building was used as an estimate of receptor distance. Figures showing staging yards and associated sensitive receptors are provided in Appendix A.

# HRA Results

The sensitive and worker MICR and HIC for each staging yard are presented in Table 3.

Table 3: Staging yard HRA Results

|  | MICR (per million persons) |  | Chronic (HIC) |  |
| --- | --- | --- | --- | --- |
| Staging Yards | Sensitive | Worker | Sensitive | Worker |
| Bear Valley | 1.493 | 0.026 | 8.56E-04 | 8.56E-04 |
| Lugo 1 | 2.107 | 0.037 | 1.21E-03 | 1.21E-03 |
| Lugo 2 | 0.500 | 0.009 | 2.87E-04 | 2.87E-04 |
| Cima Road Tree Square | 5.804 | 0.103 | 3.33E-03 | 3.33E-03 |
| Cima Road Rock Circle | 2.498 | 0.044 | 1.43E-03 | 1.43E-03 |
| Thresholds of Significance | 10.0 per million persons |  | 1.0 (unitless) |  |

All of the staging yards are located within the jurisdiction of the Mojave Desert Air Quality Management District (MDAQMD). The Air Quality Thresholds of Significance published by the MDAQMD specify a health risk significance threshold of 10 per one million persons for MICR and 1.0 for the maximally exposed individual for HIC (MDAQMD, 2020). These thresholds of significance were used to evaluate the MICR and HIC values calculated for each staging yard.

As shown in Table 3 above, the calculated MICR and HIC for each staging yard is well below the significance thresholds of 10 in one million MICR and 1.0 HIC. Therefore, the conservatively estimated health risk impacts from activities occurring at staging yards are considered less-than-significant. Detailed health risk calculations are provided in Appendix B.

Included as part of this Health Risk Assessment are the following appendices:

Appendix A – Staging Yards and Nearest Sensitive Receptors (see attached file)

Appendix B – Detailed HRA Calculations (see attached file)

Appendix C - AERSCREEN Input and Output Files (see attached file)

# References

Office of Environmental Health Hazard Assessment (OEHHA). 2015. Air Toxics Hot Spots Program Guidance Manual for Preparation of Health Risk Assessments. February. https://oehha.ca.gov/media/downloads/crnr/2015guidancemanual.pdf

Mojave Desert Air Quality Management District (MDAQMD). 2020. MDAQMD California Environmental Quality Act (CEQA) and Federal Conformity Guidelines. February.

https://www.mdaqmd.ca.gov/home/showpublisheddocument/8510/638126583450266235

# Appendix A

Staging Yards and Nearest Sensitive Receptors

# Appendix B

Detailed HRA Calculations

# Appendix C

AERSCREEN Input and Output Files

SCE on September 5, 2025, filed an Amended Application for a Permit To Construct (PTC).